Transportation Systems Analysis

Scope of Work

Noah Morland

Department of Civil & Environmental Engineering, University of Nebraska-Lincoln

CIVE 202: Civil Engineering Analysis II

Dr. Kaycie Lane

March 31, 2026

Table of Contents

1. Project Overview and Goals	3

2. Project Tasks	3

Task 1: Data Acquisition and Preprocessing	3

Task 2: Descriptive Analysis of Vehicle Characteristics	3

Task 3: Vehicle Age Distribution and Location Analysis	4

Task 4: Driving Behavior Time Series Analysis	4

Task 5: Intelligent Driver Model Simulation	4

Task 6: Documentation and Reporting	4

3. Project Deliverables	5

4. Project Schedule	5

# 1. Project Overview and Goals

The objective of this project is to analyze transportation data collected by the Federal Highway Administration using the National Household Travel Survey and the Next Generation Simulation dataset. The analysis supports transportation planning by looking at national trends in vehicle ownership and by evaluating real world driving behavior using time-based vehicle trajectory data.

The National Household Travel Survey provides information on household vehicles and travel characteristics across the United States. The Next Generation Simulation dataset contains detailed measurements of vehicle position, speed, and acceleration over time. Together, these datasets allow for analysis of transportation systems.

Specific project goals include:

• Identifying trends in vehicle type, fuel type, and vehicle age across the United States
• Comparing vehicle age across different categories and household locations
• Visualizing driving behavior using time series analysis of vehicle motion
• Simulating follower vehicle behavior using an Intelligent Driver Model
• Creating clean and clear visualizations using Python for transportation planning review

# 2. Project Tasks

## Task 1: Data Acquisition and Preprocessing

National Household Travel Survey and Next Generation Simulation datasets will be imported into Python and inspected to confirm data structure and completeness. Basic preprocessing will be performed, including removing rows with missing values in selected variables and confirming that time series data is properly ordered. A specific leader and follower vehicle pair will be selected from the simulation dataset to allow focused analysis of driving behavior.

## Task 2: Descriptive Analysis of Vehicle Characteristics

The National Household Travel Survey dataset will be used to examine vehicle ownership characteristics. Vehicle type data will be analyzed to identify the distribution of household vehicle categories such as passenger cars, sport utility vehicles, pickup trucks, and other vehicle types. Fuel type data will be examined to understand the distribution of fueling technologies such as gasoline, diesel, and hybrid vehicles.

Bar charts will be created to compare the frequency of these categories.

## Task 3: Vehicle Age Distribution and Location Analysis

Vehicle age data from the National Household Travel Survey will be used to assess how old vehicles are across the dataset. A histogram will be created to show the overall distribution of vehicle age.

Vehicle age will also be compared across different vehicles making using a boxplot to examine differences in age distributions. In addition, vehicle age will be compared between urban and rural households using a distribution-based plot to evaluate how household location relates to vehicle age. These analyses provide insight into how vehicle characteristics vary across different groups.

## Task 4: Driving Behavior Time Series Analysis

The Next Generation Simulation dataset will be used to analyze driving behavior over time. Time series plots will be created for vehicle position, speed, acceleration, and spacing between a leader and follower vehicle. Time will be used as the horizontal axis for each plot.

At least two time series plots will be included to show how follower vehicles respond to changes in leader behavior. Plot formatting such as line color, labels, and subplot organization will be adjusted using Python plotting libraries to improve clarity and presentation.

## Task 5: Intelligent Driver Model Simulation

An Intelligent Driver Model simulation will be performed using the observed leader vehicle trajectory. The simulated follower vehicle position will be calculated and compared to the observed follower vehicle position over time. This comparison will be used to evaluate how well the model represents real driving behavior.

## Task 6: Documentation and Reporting

A written report summarizing the analysis, methods, results, and interpretation will be prepared. All Python code used in the project will be documented and included in an annotated code document. Figures, tables, and data files will be organized and submitted together with the final deliverables.

# 3. Project Deliverables

Deliverables for this project include:

- Raw data files

- Python code file

- Scope of work document

- Annotated code document

- Final written report

- GitHub repository containing all project materials

# 4. Project Schedule

All deliverables will be completed and submitted to the client by March 31, 2026, at 5:00 PM CT.